# SAUNA MAINTENANCE CHECKLIST

### Quick Care for Long-Term EnjoymentAFTER EVERY USE (1 Minute)

☐ Wipe benches, backrests, and touched walls with a damp cloth
☐ Remove wet towels, rugs, buckets, and bottles
☐ Prop door open 30–60 minutes to dry the sauna or leave vents open
WEEKLY

☐ Sweep or vacuum the floor
☐ Clean benches and walls with mild cleaner
    (1:3 white vinegar + water recommended)
☐ Clean glass door with non-abrasive cleaner
☐ Wipe door handles and hardware
☐ Check air intake and exhaust vents for blockagesMONTHLY

☐ Power off heater and inspect for dust or wear
☐ Check sauna stones for cracks or crumbling
☐ Tighten bench screws and door hingesANNUALLY (or ~500 hours of use)

☐ Remove, rinse, and replace sauna stones as needed
☐ Known worn or brittle stones replaced
☐ Lightly sand benches (120–150 grit)
☐ Apply thin coat of paraffin oil to benches & walls
☐ (Wood-burning only) Have chimney/flue professionally cleaned

## 🚫 NEVER DO THIS

❌ Do not use bleach, ammonia, or harsh chemicals
❌ Do not paint, varnish, or stain interior wood
❌ Do not use high-pressure water or hoses inside the sauna
❌ Do not leave wet items inside between sessions

### Built to Last

### Leisurecraft saunas are crafted for real use.
With proper care, your sauna will deliver comfort, performance, and relaxation for decades.

Leisurecraft — Designed in Canada. Built for Real Life.